Overall Lab Objective

To create two logically separate networks (VLANs) for HR and IT departments using a single physical network interface on pfSense. You will then configure firewall rules to control the traffic between these networks, demonstrating effective network segmentation.

Prerequisites & Environment Setup

This lab requires a slightly expanded VirtualBox environment.

- pfSense VM: Your existing pfSense-Firewall VM(Main PF firewall VM).

- Two Client VMs: You need two separate client machines to test the segmentation.

  - Use your existing Client-VM and rename it to HR-Client.

  - Clone the HR-Client to create a second, identical VM and name it IT-Client. (Right-click the VM -> Clone -> Full Clone -> Give it the new name).

- Virtual Network Configuration:

  - Ensure the pfSense-Firewall VM has its LAN adapters (Adapter 3 and 4) connected to the Internal Network named hr-net and it-net, respectively.

  - Ensure both the HR-Client and IT-Client VMs have their single network adapter connected to the same hr-net and it-net Internal Network, respectively. This simulates all three devices being plugged into the same physical switch.

Lab Task 1: Creating VLANs in pfSense (HR & IT)

Goal: Define the VLAN tags that pfSense will use to separate traffic.

- Log into your pfSense web interface as the admin user.

- Navigate to Interfaces > Assignments.

- Click on the VLANs tab.

- Click the + Add button to create your first VLAN.

  - Parent Interface: Select your LAN interface (e.g., em1). This is the physical port the tagged traffic will travel on.

  - VLAN Tag: 10. This is the unique ID for the HR network.

  - Description: HR Network.

  - Click Save.

- Click + Add again to create the second VLAN.

  - Parent Interface: Select the same LAN interface (e.g., em1).

  - VLAN Tag: 20. The unique ID for the IT network.

  - Description: IT Network.

  - Click Save.

You have now defined the VLANs, but they don't do anything until they are assigned and configured.

Lab Task 2: Interface Assignments and Configuration

Goal: Turn the virtual VLANs into usable pfSense interfaces with IP addresses.

- Navigate back to Interfaces > Assignments.

- Under Available network ports, you will now see your two new VLANs.

- Click the + Add button next to VLAN 10 on em1 to create a new interface called OPT1.

- Click the + Add button next to VLAN 20 on em1 to create another new interface called OPT2.

- Now, let's configure OPT1 for the HR department. Click on the OPT1 name.

  - Check the box for Enable interface.

  - Change the Description to HR.

  - Set IPv4 Configuration Type to Static IPv4.

  - Under Static IPv4 Configuration, set the IPv4 Address to 192.168.10.1 and the subnet mask to /24.

  - Click Save, and then Apply Changes.

- Next, configure OPT2 for the IT department. Go back to Interfaces > Assignments and click on OPT2.

  - Check the box for Enable interface.

  - Change the Description to IT.

  - Set IPv4 Configuration Type to Static IPv4.

  - Set the IPv4 Address to 192.168.20.1 with a /24 subnet mask.

  - Click Save, and then Apply Changes.

Lab Task 3: DHCP Setup per VLAN

Goal: Configure pfSense to automatically assign IP addresses to devices on both the HR and IT networks.

- Navigate to Services > DHCP Server.

- You will now see tabs for your new interfaces: HR and IT.

- Click on the HR tab.

  - Check the box for Enable DHCP server on HR interface.

  - Set the Range of IP addresses to issue, for example, from 192.168.10.100 to 192.168.10.200.

  - Click Save.

- Now click on the IT tab.

  - Check the box for Enable DHCP server on IT interface.

  - Set the Range from 192.168.20.100 to 192.168.20.200.

  - Click Save.

Lab Task 4: Firewall Rule Configuration for Inter-VLAN Restrictions

Goal: Implement the core security policy: IT can access the HR network, but HR cannot access the IT network.

- First, we need to give both networks internet access.

  - Navigate to Firewall > Rules.

  - Click the HR tab. Click + Add.

  - Create a rule that allows traffic from HR net to any destination. This is a basic "allow out" rule. Click Save.

  - Click the IT tab. Click + Add.

  - Create a similar rule allowing traffic from IT net to any destination. Click Save.

- Now, create the specific inter-VLAN rule.

  - While on the IT tab, click + Add to create a new rule.

  - Action: Pass

  - Interface: IT

  - Protocol: any

  - Source: IT net

  - Destination: Select Single host or alias and type HR net.

  - Description: Allow IT to access HR network.

  - Click Save, then Apply Changes.

Important: We do NOT need to create a block rule on the HR interface. By default, pfSense blocks all traffic between interfaces unless a "Pass" rule specifically allows it. Since we never created a rule allowing HR to talk to IT, that traffic is already blocked.

Lab Task 5: Testing Segmentation with Ping and Access Attempts

Goal: Verify that your firewall rules are working exactly as intended.

- Start both client VMs: Power on HR-Client and IT-Client.

- Check IP Addresses:

  - On HR-Client, open a terminal and type ip a. It should have an IP address in the 192.168.10.x range.

  - On IT-Client, do the same. It should have an IP in the 192.168.20.x range.

- Perform the Tests from HR-Client:

  - In the terminal, test internet access: ping 8.8.8.8. This should succeed.

- Perform the Tests from IT-Client:

  - In the terminal, test internet access: ping 8.8.8.8. This should succeed.

  - Now, attempt to access the HR network: ping 192.168.10.100 (the HR-Client's IP). This should SUCCEED.

Conclusion: If your tests produce these results, you have successfully created VLANs, assigned them to interfaces, provided DHCP, and implemented an asymmetrical firewall policy to achieve network segmentation and also tested the segmentation with ping and access attempts.